# Experiment 3:Basics of FlyingDRONEwith DJI NAZA Flight Controller and FLYSKY FS-i6 Radio Transmitter.

### Basics of Flying (i) Transmitter Operation, (ii) Arming and Disarming (iii) Hover, (iv) Flying

NEXT EXPERIMENT

Flight Controller and RC Caliberation

7. SOFTWARE SIMULATION

Simulation is the imitation of the operation of a real-world process or system over time. The act of simulating something first requires that a model be developed; this model represents the key characteristics or behaviors of the selected physical or abstract system or process. The model represents the system itself, whereas the simulation represents the operation of the system over time.

We are using the NAZA MV2 Software for Drone Simulation. here below shows the software icon. Depending on the NAZA we are using we have to choose the either one of the software shown below.

STEPS TO DO SIMULATION:

- Open NAZA MV2 software

- Switch on the remote

- After opening the software, it will look like the below shown image

STEP 1: In the VIEW option we have to check the Channel monitors

In the remote we have to give the throttle as input and check the channel monitor whether the throttle is moving or not

Same way we have to give the Yaw, Pitch, Roll as inputs from the remote and check the channel monitor whether they are properly working or not

After completion of this go to the next step

STEP 2: open the Basic option in that we have aircraft, mounting, RC, Gain

AIRCRAFT:

- select the type of your drone

- click on motor test

- after clicking on that one box will open click on motor M1 and check the motor orientation

- If the motor M1 was rotating in ANTI-CLOCK wise direction, then no need to change anything

- If the motor M1 was rotating in CLOCK wise direction, then we need to change the any two cables of motor

- same process we have to do for the remaining motor

STEP 3: Now click on mounting option set it as default

STEP 4: Now click on RC option and check the sticks calibrations by giving throttle yaw radar and pitch as inputs from the remote and also check the GPS, altitude, manual modes

STEP 5: Now choose the Gain option, different aircrafts have different gains so we have to set up it manually

STEP 6: In Advanced settings first choose the motor option in that select the recommended option

STEP 7: In File safe option we have to choose the either one of option based on our choice

STEP 8: Based on our application we have to choose the intelligent orientation control option

STEP 9: By using this gimbal option we can keep the device level set it as default

STEP 10: Check the H3/4 3D option no need to change

STEP 11: Choose the voltage option and no need to change it

STEP 12: In this Limit option we have to give the values based on our requirement

STEP 13: After completion of all the above steps we have to do the basic and advanced calibrations

STEP 14: In this upgrade option we have to see the updates only

STEP 15: Just check the info option and close the software

PRACTICALS

GPS CALIBRATION

After completion of the drone making before going to the flying we have to do the GPS calibration to maintain drone stable. Before going to do calibration we have to know about GPS indications. GPS have 3 indications those are shown below             Indications:

No fly /file safe

Good to fly

Ok to fly

Procedure:

Step 1:  Switch on the Remote and Give power supply to the drone

Step 2:ON-OFF the GPS button in remote 10 times

Step 3: Observe the LED colour. If it is solid green colour it is good to fly it is a yellow colour there is some file safe errors then we have to do GPS calibrations

Step 4: First rotate the drone horizontally once and stop it exactly at starting point

Step 5:Now solid green LED will blink

Step 6:Next place the drone vertically down/up and place it at exactly at starting point

Step 7:Now the drone in stable state and ready to fly

10. How to Fly

Procedure:

Step 1: Switch on remote and give power supply to drone

Step 2: place the two sticks of remote to corners its calling arm initially it starts the motors

Step 3: Raise the throttle by 10% until the drone takeoff

Step 4: maintain stability in the drone

Step 5: slowly down the throttle to land the drone